APPROVED

TECHNICAL EXECUTIVE DIRECTOR

Doina Violeta MICU

TECHNICAL REQUREMENTS

FM TRANSMITTERS IN ACTIVE STAND-BY CONFIGURATION AND FM BAND PASS FILTERS

CPV code: 32210000-8 Radio and television equipment (Rev. 2)

Full solide state FM-VHF transmitter 2+1 @ 1kW: 7 pcs

Full solide state FM-VHF transmitter 2+1 @ 0,5kW: 1 pcs

Full solide state FM-VHF transmitter 1+1 @ 1kW: 7 pcs

FM band pass filter : 13 pcs

Broadcasting Department

Radio Broadcasting Chief  Engineer

Lorena TALPAN

Bucharest, 2022

CONTENTS

| No. | Chapter / Sub-chapter | Pag. |
| --- | --- | --- |
| 1. | INTRODUCTION | 3 |
| 2. | CONTEXT OF THE TENDER | 3 |
| 2.1. | Information about the Contracting Authority | 3 |
| 2.2. | Information about the context that led to the purchase of the products | 4 |
| 2.3. | General framework of the Contracting Authority activity sector | 5 |
| 3. | DESCRIPTION OF REQUIRED PRODUCTS | 5 |
| 3.1. | The general objective of required products | 5 |
| 3.2. | Required products and ancillary operations to be performed | 5 |
|  | 3.2.1. Products requested | 6 |
|  | 3.2.2. Warranty | 8 |
|  | 3.2.3. Delivery, packing, labelling, transport, and insurance during transport | 9 |
|  | 3.2.4. Auxiliary operations | 10 |
|  | 3.2.4.1. Training of Contracting Authority staff for product testing and operating | 10 |
|  | 3.2.4.2. Installation and commissioning | 10 |
|  | 3.2.4.3. Testing products and accessories | 10 |
|  | 3.2.4.4. Technical support and preventive maintenance during the warranty period | 11 |
|  | 3.2.4.5. Spare parts for corrective maintenance activities after warranty period expiration | 11 |
| 3.3. | Duties and responsibilities of the Parties | 11 |
| 4. | DOCUMENTATION TO BE SUPPLIED TO THE CONTRACTING AUTHORITY IN RELATION TO THE PRODUCT | 13 |
| 5. | RECEPTION OF PRODUCTS | 13 |
| 6. | PAYMENT TERMS AND CONDITIONS | 13 |
| 7. | METHODOLOGY OF BID EVALUATION | 14 |
| 8. | ANNEXES | 14 |

- INTRODUCTION

In this procedure, that will be concluded by signing a product supply contract, Romanian National Radiocommunications Company S.A. (S.N.R. S.A.) performs the role of Contracting Authority, respectively beneficiary of the contract.

The tender specifications are an integral part of the procurement documentation and constitute the minimum set of technical and quality requirements to be fulfilled, on the basis of which each bidder will elaborate the technical and economic proposal, in order to conclude the contract for the supply of products. Any annex corresponding to any chapter of this tender book, is an integral part of that chapter and implicit of the procurement documentation.

Technical offers / proposals that do not meet all the minimum requirements, specified as such in the specifications, will be declared non-compliant.  Partial tenders, in terms of quantity and quality, are not admissible, only full tenders that meet all the requirements set out in this tender book will be declared admitted.  Alternative offers are not accepted.

Any tender which deviates from the mandatory minimum requirements will be considered admissible only in so far as it ensures a higher quality level than the minimum requirements.

The documentation related to the technical proposal will be presented in English or in Romanian and will be submitted in electronic format, with text search capability. In case of submission the technical proposal in English, the bidder assumes the obligation to submit, at the request of SNR S.A., the authorized translation into Romanian language.

In the case of english technical terms and their usual acronyms, as well as those which do not have officially equivalent accepted in the Romanian language, the semantic value retains their original technical meaning (according to specifications and definitions in the terminology used by reference structures in the field of communications) throughout the description of the technical specifications in this tender book.

In this document will be used the terms Bidder and the terms Contractor or Supplier, which will have the same meaning. The terms Product Supply Contract and Contract will have the same meaning.

- PRODUCTS PURCHASE INTENTION CONTEXT

  - Information about the Contracting Authority

Name of the Contracting Authority: Societatea Naţională de Radiocomunicaţii S.A. (S.N.R. S.A.)

Address: Şoseaua Olteniței nr. 103, sector 4, Bucureşti.

  - Information about the context that led to the purchase of the products

According to the provisions of H.G. nr. 372/1998 on the establishment of the Romanian National Radiocommunications Company S.A. through the reorganization of the Autonomous Administration " Radiocomunicații ",Societatea Națională de Radiocomunicaţii (S.N.R.) S.A. is a legal Romanian entity, a national company of public interest, which operates on the basis of economic management and financial autonomy.

S.N.R. S.A. provides telecommunications services with a wide range of coverage and addressability. In order to achieve the purpose for which it is established, S.N.R. S.A. exercises its object of activity throughout the country and outside it, ensuring the transport and broadcasting of national radio and television programs, international radio and television programs, and other radio and television programs.

Romanian National Radiocommunications Company S.A. is the main provider of radio-tv broadcasting services (analog and digital), which deals with the transport and broadcasting of public radio and TV programs of the Romanian Radio and Television Broadcasting Companies at national and international level, through the largest network of DVB-T2, FM, AM and US transmitters.

Within the FM broadcast network of S.N. Radiocomunicății, several types of FM transmitters are in operation, and in the 0.5 kW-1 kW power group are the Quest type ones, which were put into operation in the period 2003-2004. These transmitters are part of a range of equipment with technology from the 1990s, and due to their age, some of the components can no longer be procured. For example, in the Huși and Vaslui sites, the transmitters had to be replaced with available spare transmitters, because their functioning became unstable and after repeated attempts to fix it, normal operation could not be restored. At Hotarele site, a good part of the electrical wiring of the transmitter was damaged by rodents, the operation being unsafe, with repeated blocking situations, random power loss, etc.

Although they are in dual exciter configuration, with automatic change over in case of failure of the main exciter, some of them no longer have this functional facility due to the fact that the switching control system is no longer working, and this component (Main Alt Controller) is " obsolete" (old component, which can no longer be used), according to the information received from the manufacturer.

As mentioned above, these equipments incorporate outdated technology from the 1990s, and the procurement of specific spare parts from the manufacturer is difficult or even impossible for certain items. Because of this, the transmitters are no longer functional at normal capacity (nominal power, technical parameters) or they no longer have the redundancy of critical components such as the exciter. The system of automatic switching in case of failure is non-functional in many of them, which no longer allows transmitter to be on air in case of failure of the main exciter.

Another aspect related to the limitations of this type of equipment is the fact that it does not have the possibility of remote monitoring/access. Because these transmitters are installed in remote locations that are not manned, difficulties arise in ensuring prompt interventions in the event of a failure. In addition, there is also the unpleasant situation that, due to the fact that there is no information about the functionality of the equipment, there are long-lasting failures, and intervention is only started after notifications or complaints being received directly from listeners or sent to Romanian Radio Company, or if any of the intervention teams have other works in the area.

The contract signed with Romanian Radio Company includes the broadcasting of Romania Actualitati and Romania Cultural programs through two networks with national coverage consisting of 116 transmitters (0.5 kW-10 kW). These networks include 23 Quest transmitters (21x1 kW transmitters and 2x0.5 kW transmitters). Taking into account the fact that the majority of Quest transmitters are in remote locations, we consider it appropriate to adopt a technical solution with active back-up, respectively: 1+1 (a main transmitter + a transmitter active bsck-up) or 2+1 (two main transmitters + one active backup transmitter).

  - General framework of the Contracting Authority activity sector

S.N.R. S.A. carries out its activity in the field of radiocommunications, the main object of activity including: the provision and marketing of communications services, maintenance, operation, development and modernization of the national system of terrestrial broadcasting for public radio and television programs, the delivery, under the law, of radiocommunications services necessary for operating the national security system.

Activity of S.N.R. S.A.  is carried out in accordance with the provisions of:

- H.G. no. 372/1998 on the establishment of S.N. Radiocomunicații S.A. through the reorganization of the Autonomous Radiocommunication Directorate;

- Law nr. 504/2002 of the audiovisual services, with subsequent amendments;

- O.U.G. nr. 111/2011 on electronic communications, with subsequent amendments;

- Law nr. 159/2016 on the regime of physical infrastructure of electronic communications networks, as well as for establishing measures to reduce the cost of deploying electronic communications networks, with subsequent amendments and additions;

- O.U.G. nr. 109/2011 on corporate governance of public enterprises, with subsequent amendments.

- DESCRIPTION OF REQUIRED PRODUCTS

  - The general objective of required products

The object of the Contract is to purchase 23 Full Solid State FM transmitters in active reserved system with a 2+1 / 1+1 configuration and 13 FM Band Pass Filters, to replace the existing ones that no longer correspond from a technical and functional point of view.

NOTE: the supply of the products will also include training in factory, factory acceptance testing, on-site acceptance testing of the products after installation by the Contracting Authority.

  - Required products and ancillary operations to be performed

In the performance of the contract, the contractor's activity shall be guided by the following principles:

- The contractor shall act in the interest of the Contracting Authority throughout the duration of the contract, under the conditions and within the limits described in the documentation relating to this procurement procedure;

- The contractor shall act in order to achieve the objective, with regard to optimising the use of resources in order to achieve the objective of the contract.

    - Requested products

Delivery of the products will be made at the following locations:

| No. | Location | Delivery address |
| --- | --- | --- |
| 1 | ALEXANDRIA | Str.Calea Dunarii nr.145, site ROMTELECOM, jud.Teleorman |
| 2 | BORSEC | Varful Teasc, Borsec, jud.Harghita |
| 3 | BRAŞOV-TÂMPA | Varful Tampa, Brasov, jud.Brasov |
| 4 | DĂBULENI | Str.Marin Preda nr.4, Dabuleni, jud.Dolj |
| 5 | GIURGIU | Bdul Mihai Viteazul nr.1, Giurgiu, jud.Giurgiu |
| 6 | HOTARELE | Com.Hotarele, jud.Giurgiu |
| 7 | MAHMUDIA | Deal Bestepe, Com.Mahmudia, jud.Tulcea |
| 8 | MOLDOVA NOUĂ | Pojejena, jud.Caras Severin |
| 9 | NEGREȘTI | Varful Jeleznic, Negresti-Oas, jud.Satu Mare |
| 10 | SIGHET-FM | Dealul Dobaies, Sighetu Marmatiei, jud.Maramures |
| 11 | VĂRATEC | Varful Varatec, Baiut, jud.Maramures |
| 12 | VASLUI | Dealul Mirenilor, Com.Tanacu, jud.Vaslui |
| 13 | HUȘI | Deal Dobrina, Husi, jud.Vaslui |
| 14 | SLOBOZIA | Str. Unirii nr.4, Slobozia, jud. Ialomița |
| 15 | SULINA | Str. Mircea Vodă nr.1, Sulina, jud. Tulcea |
| 16 | Sucursala DRc.BUCUREȘTI | Str. Carpați nr.100, sector 1, București |
| 17 | Sucursala DRc. IAȘI | Str. George Enescu nr.7, Iași, județ Iași |
| 18 | Sucursala DRc. TIMIȘOARA | Str.Aristide Demetriade nr.1, Timișoara, județ Timiș |
| 19 | Sucursala DRc. CLUJ | Str.Fagului nr.2, Cluj Napoca, județ Cluj |

The delivery Time is maximum 12 months, from the date of signing the contract.

The technical warranty of the products shall be at least 24 months from the date of the Final Acceptance Report and the certificate of acceptance on the site, as specified in point 3. 2.2. Warranty.

The products covered by the supply contract have the following technical characteristics:

- Full Solid State FM transmitters active reserved system in 2+1 / 1+1 configuration
The new equipment that will be installed will be of the latest generation, completely solid state

(transistorized).

It will be provided:

- 1+1 active reservation system (one main transmitter + one active backup transmitter) or 2+1 (two main transmitters + one active backup transmitter) and bandpass filters to limit unwanted parasitic emissions.

- Local monitoring framework and remote access interface

- Spare parts - minimum recommended set*: set of electronic modules and other components that the manufacturer considers necessary for the quick removal of failures and minimizing  downtimes.

- For sites where there are at least two transmitters, bandpass filters will be provided for the transmitters that are connected to the broadband input of the combiner or directly to the antenna.

- All equipments must be CE compliant.

- Delivery to broadcast stations.

- The transmitters will have the following main features:

- Exciter with digitally controlled oscillator and digital modulator (direct to channel)

- The transmitter will be provided with a microprocessor control system and modern user

interface with LCD display and will include all the necessary facilities for integration into Radiocom's remote monitoring system

- The transmitters will be equipped with sound processor, RDS encoder

Technical specifications of the transmitters:

| Frequency range | 87.5 ÷108 MHz software digitally programmable |
| --- | --- |
| Frequency stability | +/-150Hz |
| Nominal frequency deviation | ± 75 kHz (CCIR 450 - 1) |
| Modulation capability | ± 150 kHz; |
| Class of emission | F8EH |
| Stereo emissions | According to CCIR recommendation 450, section 2 (pilot tone procedure) |
| Output impedance | 50 Ω (Unbalanced); |
| VSWR | Automatic power reduction beyond 1.5:1, Transmitter should be protected for short and circuit conditions |
| Spurious emissions (including harmonics) a. 9kHz-1GHz b. 108 MHz-137MHz | according ETSI EN 302 018 max -16dBm (25microW) for tx 1kW better than -75dBc for tx 150W-500W max -16dBm (25microW) |
| Modulation frequency range | 30 Hz la 15 kHz; |
| Input impedance | 600 ohms, balanced for analog 110 ohms for digital |
| AF input level | 0 dBu to + 10 dBu for analog - 10 dBFS to 0 dBFS for digital; |
| Stereo Amplitude Response referred to fmod=400 Hz, 30 Hz to 15 kHz,left channel, right channel | ± 0.2 dB (without pre-emphasis) ± 0.2 dB (with pre-emphasis) |
| Crosstalk between left and right channels, 100% modulation, 30 Hz to 15 kHz | > 60 dB |
| Distortion (THD) 30 Hz - 15 kHz, left channel, right channel |  |
| - with 75 kHz frequency deviation | < 0.05% |
| - with 100 kHz frequency deviation | < 0.05%; |
| Intermodulation distortion (L , R) 60Hz/ 7kHz, 4:1, +4dBu | < 0.2% |
| FM S/N ratio, referred to fmod=400 Hz at 75 kHz frequency deviation,peak value measurement, left channel, right channel, -weighted | ≥ 80 dB |
| Asynchronous AM Noise | > 55dB |
| Synchronous AM Noise | > 50dB |
| Supply voltage | 230Vac ± 15%, 50 Hz ± 3 Hz |
| Overall efficiency | > 65% pt Tx ˂ 1kW > 70% pt Tx = 1kW |

Technical Specifications of FM Band Pass Filters

| Type | Min 3 cavities |
| --- | --- |
| Frequency range | 87.5 MHz – 108 MHz |
| Impedance | 50 ohms |
| Power | 1kW |
| Frequency | According to the frequencies specified in quotation list |
| Selectivity | Selectable +/-1.2MHz to +/-1.6MHz |
| VSWR at fc | <1.08 |
| Insertion loss | <0.5dB, selectivity +/-1.6MHz |
| Cooling | Natural heat dissipation |

Legend:

Spare parts - the minimum recommended set*- The bidder shall provide a complete list of the recommended spare parts for the post-warranty period and their price per unit. The total value of the recommended set of spare parts will not exceed 5% of the value of the transmitter.

The technical characteristics of full Solid State FM transmitters active reserved system in 2+1 / 1+1 configuration and FM Band Pass Filters that are the object of the contract, can be found in the technical proposal forms Annex 1 to this specification, respectively Annex 1a- Technical compliance for for Full solide state FM Transmitter and Annex 1b- Technical compliance for for FM Band Pass Filters.

    - Warranty

The products/ equipments delivered under the contract benefit from the warranty and technical support period assumed by the Supplier in the bidding process, of at least 24 months from the date of signing the final reception protocol and the certificate of acceptance on site, without any additional cost for the Buyer.

During the warranty period, the Contractor shall notify the Supplier within 5 days from the date of finding a malfunction, and the Supplier undertakes to repair or replace at its exclusive expense, any defective component or subassembly, including any installation material that is found to be defective or non-compliant with the technical specifications, within a maximum of 10 days from the date of notification by the Purchaser. The replaced components will have a new warranty period, which runs from the date of their replacement.

The Supplier must provide qualified service personnel capable of providing technical support and managing the fixing of the failures during the warranty period.

    - Delivery, packing, labelling, transport, and insurance during transport

The supplier shall be obliged to pack the products in such a way as to cope with its handling at loading/unloading, the conditions during transport, transit and exposure to extreme temperatures, sun and precipitations that may occur during transport and storage, so as to reach its final destination in good condition, according to the manufacturer's specifications. In the case of packing weights and volumes in boxes, the supplier shall take into account the distance to the final destination of the products.

The products will be labeled and packaged according to the provisions of the execution standards, by category (e.g. transmitters, power suplies, monitoring equipment, installation materials, FM band pass filters, etc.) in separate packages. On each package, a label containing the type of equipment contained will be displayed in a visible place. If a category of equipment is contained in several packages, the Supplier must highlight this fact and make the correlation between these packages (e.g. by numbering them).

All packaging materials for the products, as well as all the materials necessary for the protection of the packages (wooden pallets, protection sheets, etc.) remain the property of the Purchaser.

The products/ equipments will be delivered to the sites mentioned in point 3.3.1., and their installation will be carried out by the Contracting Authority. The supplier has the obligation to communicate to the Buyer, in writing, 5 days before the dispatch of the products, the shipping dates, the description of the products, the quantity. The supplier shall send to the Buyer the following documents that will accompany the products:

- Cargo accompanying notice;

- List of products contained in each package (Packing-list);

- The quality certificate;

- Warranty certificate;

- Transport document (car, sea, etc.);

- Certificate of origin of the goods;

- The factory acceptance certificate, signed by the representatives of both contracting parties.

The products will be insured against loss or damage occurred during transport and caused by any external factor, the Supplier being fully liable for any damage/ malfunction of the products occurred during transport, the latter having the obligation to remedy any deficiency occurred, possibly to replace, if necessary, the damaged / defective product according to the contractual provisions on the technical warranty.

The Supplier shall be responsible for the delivery of the products within the agreed time limit and shall be deemed to have taken into account all the difficulties which he may encounter in this respect and shall not invoke any reason for delay or additional costs.

    - Auxiliary operations

      - Training of Contracting Authority staff for product testing and operating

The contractor is responsible for the training of the personnel designated by the Contracting Authority, respectively the schooling in the factory of a number of 6 persons, during 5 working days, in order to test the technical parameters of the equipment in the factory and on the site, after installation and for the subsequent operation of the product / equipment.

The training activity will be provided at no additional cost to the Buyer. The provider will cover all costs of the training, including those related to travel and accommodation.

The technical training activities will be carried out by instructors authorized by the manufacturers of the transmitters and on their equipment; the training shall be carried out at the manufacturer's premises or in facilities approved by him, equipped in accordance with the purpose of the training.

Prior to the start of the training activities, the Tenderer has the obligation to elaborate and make available to the Buyer, in electronic format, technical-operational documentations, manuals for exploitation, administration and use in English or Romanian, for all the functionalities of the product and its equipment. The participants in the training will be provided individually with training materials on electronic and printed support. After completing the training, diplomas / certificates of participation / attestation will be issued for all participants.

      - Installation and commissioning

The on-site installation of the supplied products, will be carried out by the Buyer, under the coordination of the personnel trained by the Supplier for this purpose.

Upon completion of the installation of the transmitters on the site, the Buyer shall notify the Supplier in order to carry out, within a maximum of 15 days from receiving the notification, the on-site acceptance testing of the products and their commissioning.

      - Testing products and accessories

Testing of the full solide state FM transmitters, accessories and a FM band pass filters will be done in the factory and on site.

The products testing will be carried out in the presence of the Authorized Representatives of the Supplier and the Beneficiary's staff, trained by the Supplier in this scope.

The Contracting Authority proposes to fill in the test results in forms of the kind provided in Annex 3a and Annex 3b to this specification.

Note: It is mandatory that the submitted offer contains a detailed "Acceptance Test Protocol", including the individual procedure and test equipments used to measure and test each parameter and functionality of the transmitters.

The factory testing of the transmitters will be carried for one transmitter from each power range (2+1 FM 1kW, 2+1 FM 0.5kW, 1+1 FM 1kW), out on dummy load. Performance testing involves operation checks, electrical and mechanical checks, testing the operating regime,according to the protocol . At the end of the factory testing, the acceptance certificate will be issued, a document that will be signed by the representatives of the Beneficiary and of the Supplier.

On-site testing involves repeating factory tests on dummy load.  All tests will also be repeated in the antenna.

Accessories testing

The related equipment will be tested and inspected according to the mutually acceptable acceptance testing protocols at the manufacturer's factory as well as on site. The bidder shall attach to the submitted bid, the acceptance protocol for each type of equipment.

      - Technical support and preventive maintenance during the warranty period

Throughout the duration of the contract, the Supplier shall provide technical support.  It will provide a contact point dedicated to the Buyer, where the latter can report any problem / malfunction that requires maintenance, in order to ensure that any reported situation is promptly handled by the Supplier.

Maintenance during the warranty period, of at least 24 months, will be provided by the Supplier, at his expense, including the provision of the necessary spare parts from its own stock.  The warranty for the product(s) repaired or replaced during the warranty period is extended by the duration of their immobilization in repair.

      - Spare parts for corrective maintenance program activities after warranty period

The supplier will ensure a stock of first intervention spare parts, within the limit of 5% of the value of the product / equipment, parts that will be delivered together with the equipment and will be made available to the Buyer for use during the post-warranty period, in order to remedy with maximum urgency some possible problems.

The supplier must be able to provide spare parts and any other necessary consumables. Thus, the Bidder shall present in the proposal:

- Recommendations regarding the spare parts that must currently exist in order to facilitate the shortest possible time of the maintenance operations;

- The delivery time for the recommended spare parts, within the limit of 5% of the value of each product / equipment, is the same as the delivery term of the product;

- The way of insuring the avalability of spare parts during the post-warranty period;

- Other relevant information.

All spare parts provided by the Supplier must comply with the technical and quality requirements of the equipment manufacturer.

  - Duties and responsibilities of the Parties

- Duties and responsibilities of the Provider

- To provide products up to the standard and performance presented in its technical proposal, with the professionalism and promptness due to the commitment made;

- To inform the Buyer 5 days before the delivery of the product, in writing, by sending the documents that will accompany the product on delivery;

- To supervise the delivery of the product and the provision of related services, to ensure the human, material, equipment or other similar resources required by and for the contract;

- To ensure all activities regarding the training of the designated personnel of the Buyer;

- To check the product on departure from the factory and to ensure the proper packaging and labelling of this, according to the requirements of the specification;

- To ensure the transport of the product to the destination and the unloading of the product on site, under conditions of maximum safety;

- To insure the products during transport s;

- To deploy together with the Contracting Authority the verification / testing of the product - acceptance tests in the factory and on the site, after its installation by the Contracting Authority, within a maximum of 15 days from the receipt of the notification on the completion of the installation works, issued by the Contracting Authority;

- To provide the spare parts within the limit and the technical specification established by the contract (max. 5% of the valueof the product / equipment); their delivery will be made together with the transmitter;

- To provide technical support and free maintenance during the warranty period, to remedy within 10 days the failures notified by the Buyer;

- To solve without additional costs for the Buyer, within a maximum of 10 days from the notification of the Buyer, any inconsistency found after the quantitative acceptance, including in the case of inspected or tested products, both in the factory and on site, which do not comply with the technical specifications of the Supplier's technical proposal;

- To ensure, at the request of the Purchaser and at a reasonable price, the spare parts, materials and subassemblies necessary for the proper functioning of the delivered equipment, for a period of 10 years from the end of the contract. If during the 10 years, certain spare parts are no longer produced, the Supplier has the task of purchasing the Buyer the equivalent spare parts, at current and reasonable prices, charged on the market and accepted by the Buyer.

- Duties and responsibilities of the Buyer

- To make available to the Contractor any information and/or documents which he holds and which may be relevant to the performance of the contract;

- To designate a representative responsible for the monitoring and performance of the contract;

- To ensure the access of the Supplier to the site mentioned in this specification, in order to unload the product in conditions of maximum safety;

- To quantitatively receive the product within 5 days of its delivery on the site and to sign a quantitative reception report of, that will be aproved by both contracting parties;

- To install the product within 5 working days from the quantitative reception report and to notify the Supplier at the end of the installation of the product on the site, in order to carry out the acceptance tests within the stipulated time, together with the Supplier;

- To carry out the qualitative reception in the presence of a representative of the supplier and to draw up a performance test report, which will be signed by both contracting parties;

- To request technical support from the Supplier, under the conditions set out in the specifications and in the contract;

- To notify the Supplier within the warranty period of the product, within 5 days from the occurrence of a malfunction, in order to remedy it;

- To pay to the Supplier the price agreed for the fulfillment of the contract, based on the invoice issued by him, accompanied by the Protocol of quantity and quality reception, and by the certificates of acceptance in the factory and on the site, without observations, under the conditions mentioned in the contract.

- DOCUMENTATION TO BE SUPPLIED TO THE CONTRACTING AUTHORITY RELATED TO THE PRODUCTS

The products covered by this specification shall be compulsorily delivered together with the following documents:

- The notice accompanying the goods;

- List of products contained in each package (Packing-list);

- The quality certificate;

- Warranty certificate;

- Transport document (car, sea, etc.);

- Certificate of origin of the goods;

- Technical documentation, according to the specifications;

- Acceptance certificate, following factory testing of the product, signed by both contracting parties.

- RECEPTION OF PRODUCTS

The final reception of the products will be made at the final destination and will include cumulatively:

- The quantitative acceptance, which will consist in the inspection and visual verification by the Purchaser, counting of the products delivered, including the auxilliary components (it is mandatory to record all the series of the delivered products) within 5 working days of their receipt and a quantity reception report will be concluded, which will be signed by both contracting parties.

- Qualitative acceptance, which will involve carrying out conformity tests in order to verify compliance with the technical specifications of the product in the technical proposal.

If, during the quantitative and qualitative acceptances, it is found that not all the components of the product have been delivered, or some of the components tested by the Purchaser do not meet the technical specifications of the technical proposal or are defective, the Buyer has the right to reject the products concerned,  and the Supplier has the obligation to remedy the deficiencies found, without additional costs for the Buyer, by delivering within a maximum of 10 working days the missing product/components and/or replacing them, without exceeding the set deadline.

- PAYMENT TERMS AND CONDITIONS

The payment of the delivered products will be made based on the invoices issued by the Supplier, by bank transfer to his account, according to the provisions of the Contract.

- METHODOLOGY OF BID EVALUATION

The award criterion for the conclusion of the supply contract for products is 'the best value for money'. The score will be awarded taking into account the evaluation factors and subfactors set out in Annex 2 to this specification, respectively the financial component - the bid price and the technical component, consisting of 2 subfactors: the performance of the technical parameters and the delivery time.

The maximum score, for each factor/ subfactor, will be given to the most advantageous offer in relation to the minimum value requested.  For the rest of the admissible tenders, the score shall be calculated according to the algorithm set out in Annex 2.

The evaluation committee will award each bid an individual score, resulting from the summation of the partial scores obtained by applying the calculation algorithm for each evaluation factor/subfactor. The bid with the highest final score will be considered the winning bid.

The maximum score that an offer can accumulate is 100 points.

If two or more tenders are ranked first, with equal scores, the tiebreaker will be made taking into account the score obtained at the evaluation factors, in descending order of their weightings. If equality is maintained, the Contracting Authority shall be entitled to request new financial proposals and the successful tender shall be designated the one with the lowest financial proposal.

- ANNEXES

- Annex 1a - Technical proposal form - Technical compliance for Full solide state FM Transmitter.

- Annex 1a - Technical proposal form - Technical compliance for FM band pass filter.

- Annex 2   - Evaluation factors.

- Annex 3a - Forms for Full solide state FM Transmitter testing.

- Annex 3b - Forms for FM band pass filter transmitter testing.