Tenderer

..........................

(Company name)

Summary

Of prices for products

- FULL SOLID STATE FM-VHF TRANSMITTER 2+1 @ 1 kW

| No | FURNITURE | REQUIREMENTS | Quantity | PRICE site ALEXANDRIA | PRICE site BRAŞOV-TÂMPA | PRICE site GIURGIU | PRICE site MAHMUDIA | PRICE site NEGREŞTI | PRICE site VĂRATEC | PRICE site VASLUI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Active reserved system in 2+1 configuration | According with Technical Specifications System power: 2 x main transmitters = 1kW 1 x spare transmitter = 1kW | 1 system / site |  |  |  |  |  |  |  |
| 2 | INTERFACE FOR CONTROL and MONITORING, Local and Remote | - Local and PC remote management and monitoring facilities: hardware + software/ firmware; -All necessary devices and software to integrate the transmitter into the Radiocomunicatii monitoring system (e.g. SNMP protocol, MIB files, etc. via TCP/IP port for ethernet HTML5 web interface or software monitoring/management | 1 pc / site |  |  |  |  |  |  |  |
|  |  | Audio and modulation monitoring • FM demodulator (1 pc / main tx) • Modulation monitors and metering • Audio monitor |  |  |  |  |  |  |  |  |
| 3 | RDS ENCODERS SET suitable for 2+1 configuration: - Fully compliant with IEC standard 62106 | - Spare TX shall be able for automatically operation using RDS configuration of each of the two main transmitters - Scrolling PS and radio text messaging - PS 8 characters and up to 128 characters scrolling messaging - TCP/IP port for ethernet web GUI or software management -6 AF (alternate frequencies) at least | 1 set / site |  |  |  |  |  |  |  |
| 4 | AUDIO PROCESSORS SET suitable for 2+1 configuration | - Spare TX will be able for automatically operation using audio settings of each of the two main transmitters - Multiband processing, EQ, AGC, Clipping - processing/ bypass selectable For external device: - balanced analog input : 600 ohm/>10kohm selectable - balanced analog output: 600 ohm, balanced, (-10 dBu to +14 dBu adjustable) - balanced digital input 110 ohm - balanced digital output: 110 ohm (-20 dBFS to 0 dBFS adjustable) - auto/manual switch audio input - MPX output - XLR connectors for audio input/output - all necessary interconnecting cables | 1 set / site |  |  |  |  |  |  |  |
| 5 | TECHNICAL MANUAL AND COMPLETE DOCUMENTATION | In English | 1 pc / site |  |  |  |  |  |  |  |
| 6 | COMMISSIONING AND SITE ACCEPTANCE TESTS | No of supplier specialist & working days Supplier will support all cost for travelling, accommodation, daily expenses. Buyer will support transport from hotel to site and back | 1 pc / site |  |  |  |  |  |  |  |

TOTAL PRICE A :

- FULL SOLID STATE FM-VHF TRANSMITTER 2+1 @ 0,5 kW

| No | FURNITURE | REQUIREMENTS | Quantity | PRICE site BORSEC |
| --- | --- | --- | --- | --- |
| 1 | Active reserved system in 2+1 configuration | According with Technical Specifications System power: 2 x main transmitters = 0,5kW 1 x spare transmitter = 0,5kW | 1 system |  |
| 2 | INTERFACE FOR CONTROL and MONITORING, Local and Remote | - Local and PC remote management and monitoring facilities: hardware + software/ firmware; -All necessary devices and software to integrate the transmitter into the Radiocomunicatii monitoring system (e.g. SNMP protocol, MIB files, etc. via TCP/IP port for ethernet HTML5 web interface or software monitoring/management | 1 pc |  |
|  |  | Audio and modulation monitoring • FM demodulator (1 pc / main tx) • Modulation monitor and metering • Audio monitor |  |  |
| 3 | RDS ENCODERS SET suitable for 2+1 configuration: - Fully compliant with IEC standard 62106 | - Spare TX shall be able for automatically operation using RDS configuration of each of the two main transmitters - Scrolling PS and radio text messaging - PS 8 characters and up to 128 characters scrolling messaging - TCP/IP port for ethernet web GUI or software management -6 AF (alternate frequencies) at least | 1 set |  |
| 4 | AUDIO PROCESSORS SET suitable for 2+1 configuration | - Spare TX will be able for automatically operation using audio settings of each of the two main transmitters - Multiband processing, EQ, AGC, Clipping - processing/ bypass selectable For external device: - balanced analog input : 600 ohm/>10kohm selectable - balanced analog output: 600 ohm, balanced, (-10 dBu to +14 dBu adjustable) - balanced digital input 110 ohm - balanced digital output: 110 ohm (-20 dBFS to 0 dBFS adjustable) - auto/manual switch audio input - MPX output - XLR connectors for audio input/output - all necessary interconnecting cables | 1 pc |  |
| 5 | TECHNICAL MANUAL AND COMPLETE DOCUMENTATION | In English | 1 pc |  |
| 6 | COMMISSIONING AND SITE ACCEPTANCE TESTS | No of supplier specialist & working days Supplier will support all cost for travelling, accommodation, daily expenses. Buyer will support transport from hotel to site and back | 1 pc |  |

TOTAL PRICE B:

- FULL SOLID STATE FM-VHF TRANSMITTER 1+1 @ 1 kW

| No | FURNITURE | REQUIREMENTS | Quantity | PRICE site DĂBULENI | PRICE site HOTARELE | PRICE site MOLDOVA NOUĂ | PRICE site SIGHET | PRICE site HUSI | PRICE site SLOBOZIA | PRICE site SULINA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Active reserved system in 1+1configuration | According with Technical Specifications System power: 1 x main transmitter = 1kW 1 x spare transmitter = 1kW | 1 system/site |  |  |  |  |  |  |  |
| 2 | INTERFACE FOR CONTROL and MONITORING, Local and Remote | - Local and PC remote management and monitoring facilities: hardware + software/ firmware; -All necessary devices and software to integrate the transmitter into the Radiocomunicatii monitoring system (e.g. SNMP protocol, MIB files, etc. via TCP/IP port for ethernet HTML5 web interface or software monitoring/management | 1 pc / site |  |  |  |  |  |  |  |
|  |  | Audio and modulation monitoring • FM demodulator (1 pc / main Tx) • Modulation monitor and metering • Audio monitor |  |  |  |  |  |  |  |  |
| 3 | RDS ENCODERS SET suitable for 1+1 configuration: - Fully compliant with IEC standard 62106 | - Spare TX shall be able for automatically operation using RDS configuration of the main transmitter - Scrolling PS and radio text messaging - PS 8 characters and up to 128 characters scrolling messaging - TCP/IP port for ethernet web GUI or software management -6 AF (alternate frequencies) at least | 1 set / site |  |  |  |  |  |  |  |
| 4 | AUDIO PROCESSORS SET suitable for 1+1 configuration | - Spare TX will be able for automatically operation using audio settings of each of the two main transmitters - Multiband processing, EQ, AGC, Clipping - processing/ bypass selectable For external device: - balanced analog input : 600 ohm/>10kohm selectable - balanced analog output: 600 ohm, balanced, (-10 dBu to +14 dBu adjustable) - balanced digital input 110 ohm - balanced digital output: 110 ohm (-20 dBFS to 0 dBFS adjustable) - auto/manual switch audio input - MPX output - XLR connectors for audio input/output - all necessary interconnecting cables | 1 set / site |  |  |  |  |  |  |  |
| 5 | TECHNICAL MANUAL AND COMPLETE DOCUMENTATION | In English | 1 pc / site |  |  |  |  |  |  |  |
| 6 | COMMISSIONING AND SITE ACCEPTANCE TESTS | No of supplier specialist & working days Supplier will support all cost for travelling, accommodation, daily expenses. Buyer will support transport from hotel to site and back | 1 pc / site |  |  |  |  |  |  |  |

TOTAL PRICE C:

- SPARE PART KIT for 2+1 @ 1kW and 1+1@1 kW systems

| No | FURNITURE | REQUIREMENTS | Quantity | PRICE 1 set/Sucursala București; 1 set/Sucursala Iași; 1 set/Sucursala Timișoara; 1 set/Sucursala Cluj |
| --- | --- | --- | --- | --- |
| 1 | RECOMMENDED SPARE PARTS *The offeror will provide a complete list with the spare parts and price for each item/each set. | Each set will contain recommended minimum spare parts kit (electronic boards/modules, power supply, power amplifiers, air filters, air fan, calibrated fuse, RF power transistor, display, switching relays, etc.). The 4 sets will be identically The total value of the recommended spare parts kit will be minimum 5% of the system value of 2+1 @ 1kW and 1+1@1 kW systems | 4 Sets (1 set for each site) |  |

TOTAL PRICE D:

- SPARE PART KIT for 2+1 @ 0,5 kW system

| No | FURNITURE | REQUIREMENTS | Quantity | PRICE site Sucursala Cluj |
| --- | --- | --- | --- | --- |
| 1 | RECOMMENDED SPARE PARTS *The offeror will provide a complete list with the spare parts and price for each item/each set. | Each set will contain recommended minimum spare parts kit (electronic boards/modules, power supply, power amplifiers, air filters, air fan, calibrated fuse, RF power transistor, display, switching relays, etc.). The value of the recommended spare parts kit will be minimum 5% of the entire system value of 2+1 @ 0,5 kW system | 1 Set |  |

TOTAL PRICE E:

|  | - BAND PASS FILTER (3 CAVITIES) 1 kW |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Site | FURNITURE | REQUIREMENTS | Quantity | PRICE site |
| 1 | ALEXANDRIA | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 89,7 MHz | 1 Pc |  |
| 2 | BRAŞOV-TÂMPA | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 105 MHz | 1 Pc |  |
| 3 | DĂBULENI | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 99,1 MHz | 1 Pc |  |
| 4 | GIURGIU | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 102,6 MHz | 1 Pc |  |
| 5 | HOTARELE | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 91,1 MHz | 1 Pc |  |
| 6 | MAHMUDIA | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 102 MHz | 1 Pc |  |
| 7 | MOLDOVA NOUĂ | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 105,1 MHz | 1 Pc |  |
| 8 | NEGREŞTI | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 89,4 MHz | 1 Pc |  |
| 9 | SIGHET-FM | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 106,2 MHz | 1 Pc |  |
| 10 | VĂRATEC | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 100,8 MHz | 1 Pc |  |
| 11 | VASLUI | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 102,4 MHz | 1 Pc |  |
| 12 | HUSI | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 101,7 MHz | 1 Pc |  |
| 13 | SLOBOZIA | BAND PASS FILTER (3 CAVITIES) | 3 cavities, 1 kW, tuned to 96,3 MHz | 1 Pc |  |

TOTAL PRICE F:

G. FACTORY TRAINING AND  FACTORY ACCEPTANCE TESTS

| No | FURNITURE | REQUIREMENTS | Quantity | PRICE (EURO) |
| --- | --- | --- | --- | --- |
| 1 | FACTORY TRAINING and FACTORY ACCEPTANCE TESTS | 6 persons/ 5 working days, including: _Air Transport _Hotel Accommodation _Lunch on working day _Living expenses: 100 euro / day / person _Local transport _Insurances | 1 Set |  |

TOTAL PRICE G:

| Frequency List |  |  |
| --- | --- | --- |
| Site | Frequency | System type |
| ALEXANDRIA | 91,8 MHz | 2+1 |
| ALEXANDRIA | 89,7 MHz |  |
| BORSEC | 101,00 MHz | 2+1 |
| BORSEC | 98,40 MHz |  |
| BRAŞOV-TÂMPA | 102,5 MHz | 2+1 |
| BRAŞOV-TÂMPA | 105 MHz |  |
| DĂBULENI | 99,10 MHz | 1+1 |
| GIURGIU | 104,6 MHz | 2+1 |
| GIURGIU | 102,6 MHz |  |
| HOTARELE | 91,1 MHz | 1+1 |
| MAHMUDIA | 100,5 MHz | 2+1 |
| MAHMUDIA | 102 MHz |  |
| MOLDOVA NOUĂ | 105,10 MHz | 1+1 |
| NEGREŞTI | 89,40 MHz | 2+1 |
| NEGREŞTI | 93,3 MHz |  |
| SIGHET-FM | 106,20 MHz | 1+1 |
| VĂRATEC | 91,20 MHz | 2+1 |
| VĂRATEC | 100,80 MHz |  |
| VASLUI | 106,10 MHz | 2+1 |
| VASLUI | 102,40 MHz |  |
| HUSI | 101,7 MHz | 1+1 |
| SLOBOZIA | 96,3 MHz | 1+1 |
| SULINA | 100,9 MHz | 1+1 |

| Quotation Grand TOTAL |  |  |
| --- | --- | --- |
| Item | Description | Price |
| Total price A | FULL SOLID STATE FM-VHF TRANSMITTER 2+1 @ 1 kW |  |
| Total price B | FULL SOLID STATE FM-VHF TRANSMITTER 2+1 @ 0,5 kW |  |
| Total price C | FULL SOLID STATE FM-VHF TRANSMITTER 1+1 @ 1 kW |  |
| Total price D | SPARE PART KIT for 2+1 @ 1kW and 1+1@1 kW systems |  |
| Total price E | SPARE PART KIT for 2+1 @ 0,5 kW system |  |
| Total price F | BAND PASS FILTER (3 CAVITIES) 1 kW |  |
| Total price G | Factory training and Factory acceptance tests |  |
|  | TOTAL |  |